| CHUKA |  | UNIVERSITY |
| --- | --- | --- |

UNIVERSITY EXAMINATIONS

RESITS/SPECIAL EXAMINATIONS

EXAMINATION FOR THE AWARD OF DEGREE OF BACHELOR OF SCIENCE IN APPLIED COMPUTER SCIENCE

ACSC 102: INTRODUCTION TO DIGITAL LOGIC

STREAMS:	Y1S1					   		              TIME: 2 HOURS

DAY/DATE: TUESDAY 29/8/2023 			                   8.30 A.M. – 10.30 A.M.

Instruction: Answer question one and any other two questions.

Question One (30 marks)

- Explain the role of the following registers in the processor.

- Program Counter (PC)							(2 marks)

- Memory Address Register (MAR)					(2 marks)

- Perform the following number system conversion.

597.48510 to Binary system. 						(4 marks)

- Find the sum of 3910 and -1710 in binary using the two’s complement arithmetic. Use 8 bits to represent the binary numbers. 						(4 marks)

- Construct a truth table for the Boolean expression shown below.		(4 marks)

- Briefly explain the fetch-execute cycle in instruction execution. 		(4 marks)

- Simplify the Boolean function using Boolean identities. Show the Boolean identities used in each step.										(5 marks)

- Draw the combinational circuit that directly implements the following Boolean function. 												(5 marks)

Question Two (20 marks)

- Discuss two ways of dealing with multiple interrupts in the processor.        (4 marks)

- Design a truth table for a three –input exclusive-OR (XOR) operation. Design its implementation using AND, OR and NOT gates.                                  	(8 marks)

- Get the simplified version of the Boolean function represented in the Kmap shown below.												(8 marks)

Question Three (20 marks)

- Create a Kmap and then simplify the following function.			(8 marks)

A three-input digital circuit gives a TRUE output when a majority (i.e. 2 or more) of the inputs is TRUE.

- Develop a truth table for the output 					(4 marks)

- Draw the logic diagram for the circuit implementation using AND, OR and NOT gates.  									(8 marks)

Question Four (20 marks)

- Describe the following cache write policies.					(4 marks)

- write-through

- Write-back

- Show that	  using Boolean identities.			(8 marks)

- Describe the four major internal structural components of a computer system. Use a well labeled diagram to illustrate their interconnection.				(8 marks)

Question Five (20 marks)

- Discuss two memory access techniques used in the computer system.		(4 marks)

- Discuss the flow of program execution in the event of a raised interrupt when interrupts are enabled. Use an appropriate diagram to illustrate your answer.			(8 marks)

- Perform the following number conversions:

- 543.1510 to base 8							(4 marks)

- 8F4A216 to binary 							(4 marks)

………………………………………………………………………………………………….