THIRD YEAR EXAMINATION FOR THE DEGREE OF BACHELOR OF SCIENCE IN ELECTRICAL ENGINEERING

EENG 362: RENEWABLE ENERGY SYSTEMS EXAM

Time: 2 hours

Instructions.

- This paper consist four questions

- Attempt Question one and any other two.

QUESTION ONE (30 MARKS)

a). Explain the significance of the bandgap energy in relation to the photovoltaic effect.  (2 marks)

b). i). With various expressions, distinguish between Spectral Irradiance, Insolation, and Radiation

(5 marks)

ii). A photovoltaic module that has an area of 0.3 m2 receives a light whose spectral irradiance is 1000(W/m2 μm) limited over the wavelength from λ 0.60 to 0.7 μm. Calculate the insolation received and the radiation after one day.                                                                         (4 marks)

c). Discuss two main advantages and disadvantages of tidal power systems.                        (4 marks)

d). Discuss the significance of the blade length and the number of blades in a HAWT paying attention to the output power and application of the turbine.                                (4 marks)

e). i).  Explain why biomass is, in principle, a solar source of energy.                   (4 marks)

ii). Calculate the volume of air-dried wood that is needed to bring one liter of pure water from 25°C to the boiling point (i.e., to 100°C). Given that the specific heat of water = 4,200 J /kg K, the mass of 1 liter of water is 1 kg, the energy density of air-dried wood is 15 MJ/ kg, and the density of air-dried wood is 600 kg/ m−3.                                                                               (4 marks)

f). Discuss the shortcomings of using straw as the main fuel for a power generation plant.  (3 marks)

QUESTION TWO (20 MARKS)

a). Discuss four factors that affect the efficiency of a solar cell.                                   (4 marks)

b). A solar cell has a reverse saturation current of I0 10–10 A and a short-circuit current of 5A at full insolation and room temperature. Given that its ideality factor is unity, determine its;

- Open-circuit voltage at full insolation                                                                     (3 marks)

- The percentage change in the open-circuit voltage if the insolation is reduced by 50%.

(3 marks)

- Comment on the results of b). ii. Above.                                                             (2 marks)

c). With a neat structure, discuss the operation of a generic PV cell                      (8 marks)

QUESTION THREE (20 MARKS)

a). With the use of sketch diagrams, explain the difference between a drag and a lift type wind turbine.                                                                                                                       (6 marks)

b). A proposed hydropower plant is to provide an annual energy output of 400 GWh. Determine the required rated capacity if the plant is to supply;

- Base load only

- Peak load with 20% capacity factor (CF).                                                       (6 marks)

c). Sketch a typical power-speed curve of a wind turbine and use it to define the following terms: cut-in speed, rated speed, shutdown speed, and rated power of a wind turbine.           (8 marks)

QUESTION FOUR (20 MARKS)

a). Highlight three primary forms of Direct Use of Geothermal Energy              (3 marks)

b). with the aid of a diagram explain the principle of electrical power generation in a hydroelectric plant.                                                                                                                     (6 marks)

c). A tidal stream turbine has an efficiency of 20% and generates an output of 50 kW in a flow velocity of 2.5 (m/s). Calculate the diameter of the turbine. You may assume that the density of water is ρ 1000 kg/m3.                                                                                   (6 marks)

d). Using appropriate diagrams, explain the operation of a single basin tidal barrage power plant and explain why such power plants have difficulties in meeting typical variations in power demand during the course of a day.                                                                                         (5 marks)