CHUKA	        			                               UNIVERSITY

Knowledge is Wealth (Sapientia divitia est)

Telephones: 020 2310512/18 							P. O. Box 109-60400, Chuka

Email: info@chuka.ac.ke							Website: www.chuka.ac.ke

DEPARTMENT OF PHYSICAL SCIENCES

EXAMINATION FOR THE AWARD OF BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONIC ENGINEERING

EENG 394: DIGITAL ELECTRONICS

INSTRUCTIONS:

Answer question ONE and any other TWO questions

Do not write on the question paper

QUESTION ONE

- Differentiate between combinational and sequential circuits (2 Mks)

- Define the following terms as used in digital electronics (4 Mks)

- Canonical form

- Logic gates

- Flip-flop

- Karnaugh map

- How do you implement three-input and four-input EX-OR logic functions with the help of two-input EX-OR gates? (3 Mks)

- Convert the decimal number 2497.5010 to its equivalent octal (4 Mks).

- Typically, digital thermometers use BCD to drive their digital displays (3 Mks).

- How many BCD bits are required to drive a 3-digit thermometer display?

- What 12-bits are sent to display for a temperature of 157 ºC?

- Use a Karnaugh map to minimize the following POS expression (4 Mks)

- Minimize the following function and realize the minimized function using NOR gates (4 Mks)

F (A, B, C, D) = BC + A B+ BCD + ABD + ABCD

- Simplify the following expression using De Morgan’s theorems [3 Mks].

A (B+C) D

- Minimize the given Boolean expression using K-map (3 Mks).

F(A, B, C, D) = m (3, 4, 5, 7, 9, 13, 15)

QUESTION TWO

a.

- Explain three characteristics of CMOS logic circuit (3 Mks)?

- Explain the operation of a CMOS NAND gate (4 Mks)

- A binary counter is being pulsed by a 512 kHz clock signal. The output frequency from the last flip-flop is 4 kHz. Determine

- the MOD number and (2 Mks)

- the counting range (2 Mks).

- An electrical control system uses three positional sensing devices, each of which produce 1 output when the position is confirmed. These devices are to be used in conjunction with a logic network of AND and OR gates and the output of the network is to be 1 when two or more of the sensing devices are producing signals of 1. Draw a network diagram of a suitable gate arrangement. (5 Mks)

- Design a combinational logic circuit for the given Boolean function using a Decoder (4 Mks)

QUESTION THREE

  - Explain the difference between multiplexer and demultiplexer (1 Mk)?

  - Describe the operation of a 8 to 1 multiplexer (4 Mks).

- An Engineer require a binary ripple counter that counts 000 and 111 and skips the remaining six states, that is, 001, 010, 011, 100, 101 and 110. Using a presentable, clearable negative edge-triggered J-K flip-flops with active LOW PRESET and CLEAR inputs,

- Design a binary ripple counter (3 Mks)

- Draw the timing waveforms (2 Mks)

- Determine the frequency of different flip-flop outputs for a given clock frequency, f (1 Mk)

- Using a K-map simplify the following function and realize using NAND gate (6 Mks)

F (A, B, C, D) = (1, 4, 6, 7, 8, 9, 10, 11, 15)

- Explain three uses of Counters in digital systems (3 Mks)

QUESTION FOUR

- Design a full adder circuit from half adders starting with a truth table and the resulting equations (10 Mks).

- Draw the circuit of gates that would effect the function

F = A+B.C

Simplify this function and hence redraw the circuit that could effect it (6 Mks)

- Compare and contrast the differences between analog and digital signals (4 Mks)