| CHUKA |  | UNIVERSITY |
| --- | --- | --- |

UNIVERSITY EXAMINATIONS

EXAMINATION FOR THE AWARD OF DEGREE OF BACHELOR OF SCIENCE IN PHYSICS AND BACHELOR OF EDUCATION (SCIENCE)

PHYS 476: ANALOG ELECTRONICS

STREAMS:				   		  		        	          TIME:2 HOURS

DAY/DATE:  THURSDAY 13/04/2023 			           	              11.30 A.M. –1.30 P.M.

INSTRUCTIONS

Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- What do you understand by transistor biasing?					        [3 marks]

- An npn silicon transistor has Vcc=5V and the collector load Rc=2 kΩ find,

i) the maximum collector current that can be allowed during the application of signal for faithful application							      	        [3 marks]

ii) the minimum zero signal collector current required			        [2 marks]

- What do understand by multistage transistor amplifier?			                    [2 marks]

- The figure below shows a silicon resistor with β=50 biased by base resistor method

- Draw the dc load line

- Determine the operating point

- What is the stability factor?          			                                [8 marks]

- i) Determine the type of biasing in the figure below				          [1 mark]

ii) Find its operating point							        [5 marks]

- Describe characteristics of OPAMP						        [3 marks]

- Explain the advantages of using silicon and not germanium in semiconductor devices										    			        [4 marks]

QUESTION TWO (20 MARKS)

a) A base bias circuit in the figure below is subjected to an increase in temperature from 25°C to 75°C. If β = 100 at 25°C and 150 at 75°C, determine the percentage change in Q-point values (VCE and IC) over this temperature range. Neglect any change in VBE and the effects of any leakage current									  		        [8 marks]

b) In the circuit shown below, the operating point is chosen such that IC =2mA, VCE = 3V. If RC = 2.2 kΩ, VCC = 9V and β = 50, determine the values of R1, R2 and RE. Take VBE = 0.3V and I1 = 10IB.							       			      	        [7 marks]

c) For the emitter bias circuit shown below, find IE, IC, VC and VCE for β = 85

and VBE = 0.7V.									        [5 marks]

QUESTION THREE (20 MARKS)

a) The circuit shown below uses silicon transistor having β = 100. Find the operating point and stability factor      								    	        [9 marks]

b) An npn transistor circuit below has α = 0.985 and VBE = 0.3V. If VCC =16V, calculate R1 and RC to place Q point at IC = 2mA, VCE = 6 volts.					                    [7 marks]

c) Find the value of IC for potential divider method if VCC = 9V, RE = 1kΩ, R1 = 39 kΩ, R2 = 10 kΩ, RC = 2.7 kΩ, VBE = 0.15 V and β = 90.					                    [4 marks]

QUESTION FOUR (20 MARKS)

- sketch the h-parameter equivalent circuit

(b) determine the amplifier current and voltage gains using

(i) network analysis, and

(ii) h-parameter equations.

The h-parameters are hie =2.5 kΩ ; hfe =90; hoe =45 S				      [15 marks]

c) A transistor employs a 4 KΩ load and VCC=13V. What is the maximum input signal if β=100? Given Vknee=1V and a change of 1V in VBE causes a change of 5mA in collector current    [5 marks]

QUESTION FIVE (20 MARKS)

a) The figure below shows a circuit of a summing amplifier. Determine the value of output voltage for this circuit						                                      	      [10 marks]

2 V	10KΩ

10 KΩ	+Ve

6 V		Vout

5 V	10KΩ	-Ve

b) A differentiator has a feedback resistor R1=14 KΩ and input capacitance C1=0.02µF. Calculate cut-off frequency and its maximum linear operating frequency                     	        [4 marks]

c) The figure below shows an OPAMP configuration. Determine the value of R2 required to produce a gain of 150							                                [6 marks]

R2

+ve

R1=2.4 KΩ	Vout

Va	-ve

---------------------------------------------------------------------------------------------------------------------