| CHUKA |  | UNIVERSITY |
| --- | --- | --- |

UNIVERSITY EXAMINATIONS

FIFTH YEAR EXAMINATION FOR THE AWARD OF DEGREE OF BACHELOR OF SCIENCE IN ELECTRICAL ENGINEERING

EENG 566: ALTERNATIVE ENERGY

STREAMS: 							    	 	          TIME:2 HOURS

DAY/DATE:  THURSDAY 13/04/2023 			           	                2.30 P.M. –4.30 P.M.

INSTRUCTIONS

- This paper consist four questions

- Attempt Question one and any other two.

QUESTION ONE (30 MARKS)

a). Discuss the photovoltaic effect.   							        (3 marks)

b). Explain the significance of the bandgap energy in relation to the photovoltaic effect.  (3 marks)

c). i). For a silicon based solar cell, determine the minimum frequency a photon must have to be able to create an electron-hole-pair.                                                                                    (3 marks)

ii). calculate the corresponding maximum wavelength for (c). i). above.                      (3 marks)

d). i). With various expressions, distinguish between Spectral Irradiance, Insolation, and Radiation

(4 marks)

ii). A photovoltaic module that has an area of 0.3 m2 receives a light whose spectral irradiance is 1000(W/m2 μm) limited over the wavelength from λ 0.60 to 0.7 μm. Calculate the insolation received and the radiation after one day.                                                                  	        (4 marks)

e). Give an expression of air mass ratio and explain its significance to the photovoltaic effect.

(4 marks)

f). A PV module is made up of 36 identical cells, each having a parallel resistance of 6 ohms and a reverse saturation current of I0=5X10-10A. On average, the module provides 4A for 7 hours each day. The module is connected without a blocking diode to a battery with voltage 12.7V.            										        (6 marks)

- Estimate the ampere-hours that will be discharged from the battery over a 12-hour night and the energy in watt-hours that will be lost due to this discharge.

- If a blocking diode is added, how much energy will be dissipated through the diode during the daytime assuming the on-state voltage of the diode is 0.6V?

QUESTION TWO (20 MARKS)

a). Explain with a block diagram why a power processing system is normally needed as an interface in a PV generating system.          						        (6 marks)

b). A PV module made from crystalline silicon has an open-circuit voltage of

21.5V and a maximum power of 120W at STC. The module has a NOCT of 45°C. Estimate for this module under insolation G=1000K/m2 and ambient temperature of 30°C.          (9 marks)

- the cell temperature

- the open-circuit voltage

- the maximum power output

c). Explain the significance of blocking diodes in PV modules and arrays.                      (5 marks)

QUESTION THREE (20 MARKS)

a). Draw a block diagram of grid-connected a PV system and explain the function of each block.

(6 marks)

b). A permanent magnet DC motor has an armature resistance of 1 ohm. When it is spinning at its rated speed of 52.4 rad/s, the back EMF is 10V and the armature current is 2A. Determine;

(8 marks)

- The motor applied voltage

- The motor back EMF constant

- The electrical input and mechanical output powers

- The steady-state torque.

c). A 200-Ah, 12V battery with an open-circuit voltage of 12.4V at its current SoC is charged at a C/5 rate with a charging voltage of 13.5V. Determine:                                                      (6 marks)

- The internal resistance of the battery.

- The percentage of the battery input power dissipated as heat in the internal resistance of the battery

- The percentage of the battery input power dissipated as heat in the internal resistance of the battery if the battery is charged at a C/20 rate.

QUESTION FOUR (20 MARKS)

a). Discuss four factors that affect the efficiency of a solar cell.                                            (4 marks)

b). A solar cell has a reverse saturation current of I0=10-10A . A and a short-circuit current of 5A at full insolation and room temperature. Given that its ideality factor is unity, determine its;

- Open-circuit voltage at full insolation                                                                       (3 marks)

- The percentage change in the open-circuit voltage if the insolation is reduced by 50%.

(3 marks)

- Comment on the results of b). ii. Above.                                                                   (2 marks)

c). With a neat structure, discuss the operation of a generic PV cell                                      (8 marks)

---------------------------------------------------------------------------------------------------------------------