| CHUKA |  | UNIVERSITY |
| --- | --- | --- |

UNIVERSITY EXAMINATIONS

FIFTH YEAR SECOND SEMESTER EXAMINATION FOR THE AWARD OF DEGREE OF BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONIC ENGINEERING

EPHY 251: ELECTRICAL NETWORKS I

STREAMS:		    		   		  		        	          TIME:2 HOURS

DAY/DATE:  THURSDAY 13/04/2023 			           	              11.30 A.M. –1.30 P.M.

INSTRUCTIONS

 Answer question one and any other two questions

 Do not write on the question paper

# QUESTION ONE (30 MARKS)

- Define Kirchhoff’s Current Law (KCL)                                             		(1 Mark)

- Give two advantages of circuit analysis in the frequency domain 	          (2 Marks)

- Distinguish between the following terms in electrical circuit and network analysis

(2 Marks)

- Loop

- Mesh

- Describe what you understand by the term a port and briefly discuss some of the reasons why we should study two-ports and the parameters that describe them.                (3 Marks)

- Using phasors, determine i(t) in the following integro-differential equation. Also, find the average and the r.m.s values of i(t).                      			           (6 Marks)

- Find the current i(t) flowing into the network in Figure Q1(f) using phasors.       (6 Marks)

Figure Q1(f)

- For the following voltage and current phasors below, calculate the complex power, apparent power, real power, and reactive power. Specify whether the power factor is leading or lagging.                                       			     	       (10 Marks)

# QUESTION TWO (20 MARKS)

- Find the Fourier transform of the switched on exponential function shown in figure Q2(a) below                                                                   				           (4 Marks)

Figure Q2 (a)

- Find the value of the resistance between terminals a-b of the network shown in figure Q2 (b) below                                                        				           (4 Marks)

Figure Q 2(b)

- A 20μF capacitor is connected in series with a 50kΩ resistor and the circuit is connected to a 20V, dc supply. Determine                   				           (5 Marks)

  - Initial value of current flowing

  - The time constant of the circuit

  - The value of current one second after connection

  - Value of the capacitor two seconds after connection

  - Time after connection when resistor voltage is 15V.

- Find the y parameters for the resistive network shown in Figure Q2 (d) below          (7 Marks)

Figure Q 2(d)

# QUESTION THREE (20 MARKS)

- Discuss two limitations of Ohm’s law                                       	           (2 Marks)

- By inspection, determine:                                                                                 (3 Marks)

- A circuit, having a resistance of 4.0Ω with an inductance of 0.5H and a variable capacitance in series, is connected across a 100V, 50Hz supply.

Calculate:                                               		                                                        (5 Marks)

    - the capacitance required to attain resonance;

    - voltages across the inductance and the capacitance at resonance;

    - the Q factor of the circuit.

- A 208-V three-phase power system is shown in Figure Figure Q 3(d). It consists of an ideal 208-V, Y-connected three-phase generator connected through a three-phase transmission line to a Y-connected load. The transmission line has an impedance of (0.06 + j0.12) Ω per phase, and the load has an impedance of (12 + j9) Ω per phase. For this simple power system, find                                                                                 	         (10 Marks)

  - The magnitude of the line current IL

  - The magnitude of the load's line and phase voltages VLL and VΦL

  - The real, reactive, and apparent powers consumed by the load

  - The power factor of the load

  - The real, reactive, and apparent powers consumed by the

transmission line  vi. 	The generator’s power factor.

Figure Q 3(d)

# QUESTION FOUR (20 MARKS)

- Define a phasor                                                                                                        (1 Mark)

- Briefly explain what you understand by the term mutual inductance                     (1 Mark)

- A coil of 5Ω resistance and inductance of 100mH is connected in series with a

capacitance of 2μF and a resistance of 5Ω across a 50V, variable frequency supply. Determine                                                          				           (5 Marks)

i) The resonant frequency

ii) The current at resonance

iii) Voltages across the coil and capacitor at resonance  iv) The Q factor of the circuit

- Calculate the phasor currents I1 and I2 in the circuit of Figure Q4 (d) below     (4 Marks)

Figure Q 4(d)

- Applying Kirchhoff’s current law, determine the current i2 for the network shown in figure 4 (e) below                                                           		                       (4 Marks)

Figure Q 4(e)

- Determine the steady-state current i(t) in the RLC circuit shown in figure Q 4(f) using phasors and impedances                                     		                                   (5 Marks)

Figure Q 4(f)

---------------------------------------------------------------------------------------------------------------------